3D Visualization & Analytics · ArcGIS Pro SDK · ArcGIS Maps SDK for JavaScript

# Turning Shadows into Data

From Visual Effect to Persistent GIS Feature Layer

By  Ariful Islam   ·   GIS Developer  ·  ArcGIS Pro SDK  ·  Maps SDK for JavaScript

April 2025   ·   10 min read

Have you ever wished that the shadows ArcGIS renders could be stored, queried, and reused across your GIS platform — not just viewed? This post shows you exactly how to do it.

## Introduction

Shadow analysis is one of the most visually compelling capabilities in ArcGIS. Urban planners use it to evaluate sunlight access, architects use it to simulate building impact before construction, and environmental consultants use it to assess pedestrian comfort in dense city neighborhoods.

But there is a problem that rarely gets discussed: every shadow you have ever generated in ArcGIS disappeared the moment you closed the scene.

The ShadowCast widget in the ArcGIS Maps SDK for JavaScript is a powerful tool — recently upgraded to a full analysis object. Yet its output is still a temporary visual overlay in a SceneView. It is not stored, not queryable, and not reusable across applications.

“What if a shadow could be stored in a geodatabase, queried with SQL, and consumed by any GIS application — just like a feature layer?”

That is the question this project explores. Instead of treating shadows as a rendering effect, we generate them as persistent geospatial polygons using the ArcGIS Pro SDK for .NET, and then load them as a standard FeatureLayer in the ArcGIS Maps SDK for JavaScript. The result is a shadow dataset that can be queried, filtered by time, shared across teams, and integrated into any workflow that consumes feature data.

|  | Why this is genuinely new The ArcGIS ecosystem has shadow visualization tools. It has never had a documented workflow for generating shadow geometry as persistent, queryable GIS features using the Pro SDK and consuming them cross-platform via the JS SDK. This is that workflow. |
| --- | --- |

📸 Figure 1 — The Gap — ShadowCast Widget (Temporary Visualization) Show the ArcGIS Maps SDK for JavaScript ShadowCast widget in a 3D NYC scene with shadow overlays active. How to capture: Open https://developers.arcgis.com/javascript/latest/sample-code/widgets-shadowcast/ Set date/time to June 21, 9 AM. Screenshot the full 3D scene at 1920×1080. Caption: “The ShadowCast widget renders shadows as a temporary visual overlay — not storable as GIS data.”

Figure 1 — ArcGIS ShadowCast widget showing cumulative shadow analysis. Output is visual-only — not storable as GIS data.

## Free Data Sources for This Workflow

Before writing a single line of SDK code, you need building footprints with height attributes. Below are four freely available, production-quality datasets you can use directly with this workflow — no licensing fees required.

| Source | Dataset | Description & Download |
| --- | --- | --- |
| NYC OpenData | NYC Building Footprints | The gold standard for urban shadow analysis. Every NYC building with footprint geometry, roof height, ground elevation, construction year, and BIN. Updated weekly. Directly loadable in ArcGIS Pro. Download / Explore → heightroof · groundelev · bin · cnstrct_yr |
| Google Open Buildings | Open Buildings 2.5D Temporal | AI-derived building heights at 4m resolution from Sentinel-2 imagery, covering most of the globe (2016–2023). Combine with V3 polygon footprints for a complete feature dataset. Download / Explore → height_m · year · confidence |
| OpenStreetMap | OSM via GeoFabrik | Global coverage. Many urban areas include building:height or building:levels tags. Best for rapid prototyping. ArcGIS Pro can load GeoJSON directly via the Add Data dialog. Download / Explore → building:height · building:levels |
| Microsoft | US Building Footprints | 125 million building footprints for the continental US from Bing Maps deep learning. Geometry only — no heights. Pair with USGS 3DEP elevation service to derive building heights. Download / Explore → geometry (Bing derived) · 125M buildings |

|  | Recommended starting point For your first run, use the NYC Building Footprints filtered to Manhattan Community Board 5 (Midtown). It contains ~7,000 buildings with roof heights — manageable in size, dense enough to produce compelling shadow overlap patterns. Download the Shapefile, load it in ArcGIS Pro, and connect to the add-in. |
| --- | --- |

## The Conceptual Shift

The core idea is simple but has surprisingly large implications. Shadow geometry is, at its heart, a spatial operation: you project a polygon (a building footprint) in a direction determined by the sun angle, producing another polygon (the shadow footprint). That shadow polygon is geometry. And geometry is GIS data.

Instead of asking a rendering engine to draw the shadow, we ask the geometry engine to construct it — and then we save it.

## Why This Has Never Been Documented

The Esri community has asked about programmatic shadow geometry before. A thread in the ArcGIS Pro SDK forums specifically asks: "Is there any way to obtain the shadow range?" — and the answer, as of that post, was effectively no. There is no built-in API for extracting shadow geometry from ArcGIS Pro as a feature class.

This workflow fills that gap. Instead of waiting for a native API, we compute shadow geometry ourselves using solar position mathematics and the Pro SDK's geometry engine, then write results directly to a feature class.

|  | What existing ArcGIS tools do NOT provide The ShadowCast widget (JS SDK) renders visual overlays. ArcGIS Pro's Daylight tool animates sun position. Neither produces storable, queryable shadow polygons. This project introduces that capability as a developer workflow. |
| --- | --- |

### Comparison with Existing ArcGIS Shadow Capabilities

| Capability | ShadowCast Widget (JS SDK) | Pro Daylight Tool | This Project |
| --- | --- | --- | --- |
| Visual shadow overlay | ✔ | ✔ | ✔ |
| Persistent storage in GDB | ✘ | ✘ | ✔ |
| Queryable with SQL | ✘ | ✘ | ✔ |
| Time-series shadow dataset | ✘ | ✘ | ✔ |
| Cross-platform enterprise reuse | ✘ | ✘ | ✔ |
| Spatial join with other layers | ✘ | ✘ | ✔ |

## Implementation Walkthrough

The workflow runs in four phases: solar position calculation, shadow polygon generation, feature class persistence, and web visualization with temporal filtering.

### Step 1 — Computing Sun Position (C# / ArcGIS Pro SDK)

For a given latitude, longitude, date, and time, we compute the sun's azimuth (horizontal angle from north) and altitude (angle above the horizon). These two values define the direction and length of every shadow cast.

SunCalculator.cs — ArcGIS Pro SDK

public class SunCalculator { // Returns solar azimuth (deg from N) and altitude (deg above horizon) public static (double azimuth, double altitude) GetSunPosition( double latitude, double longitude, DateTime utcDateTime) { double dayOfYear = utcDateTime.DayOfYear; double B = (2 * Math.PI / 365) * (dayOfYear - 1); // Solar declination double decl = 0.006918 - 0.399912 * Math.Cos(B) + 0.070257 * Math.Sin(B); // Hour angle double solarNoon = 12.0 - (longitude / 15.0); double hourAngle = (15.0 * (utcDateTime.Hour + utcDateTime.Minute / 60.0 - solarNoon)) * (Math.PI / 180); double lat = latitude * (Math.PI / 180); // Solar altitude double sinAlt = Math.Sin(decl) * Math.Sin(lat) + Math.Cos(decl) * Math.Cos(lat) * Math.Cos(hourAngle); double altitude = Math.Asin(sinAlt) * (180 / Math.PI); // Solar azimuth double cosAz = (Math.Sin(decl) - Math.Sin(lat) * sinAlt) / (Math.Cos(lat) * Math.Cos(Math.Asin(sinAlt))); double azimuth = Math.Acos(Math.Clamp(cosAz, -1, 1)) * (180 / Math.PI); if (hourAngle > 0) azimuth = 360 - azimuth; return (azimuth, altitude); } }

### Step 2 — Generating Shadow Polygons (ArcGIS Pro SDK Geometry Engine)

Each building footprint is projected opposite to the sun direction, scaled by height divided by the tangent of solar altitude. This produces the shadow footprint on the ground plane. Unioning the original footprint with its offset fills the full shadow area.

ShadowGeneration.cs — ArcGIS Pro SDK

public static Polygon GenerateShadowPolygon( Polygon buildingFootprint, double buildingHeightMeters, double solarAzimuthDeg, double solarAltitudeDeg) { // Shadow length = height / tan(altitude) — longer when sun is low double altRad = solarAltitudeDeg * (Math.PI / 180); double shadowLength = buildingHeightMeters / Math.Tan(altRad); // Direction vector opposite to the sun double azRad = solarAzimuthDeg * (Math.PI / 180); double dx = -Math.Sin(azRad) * shadowLength; double dy = -Math.Cos(azRad) * shadowLength; GeometryEngine engine = GeometryEngine.Instance; Polygon offsetPolygon = (Polygon) engine.Move(buildingFootprint, dx, dy); // Union footprint + offset to fill the complete shadow area Geometry shadow = engine.Union(buildingFootprint, offsetPolygon); return shadow as Polygon; }

📸 Figure 2 — ArcGIS Pro — Building Footprints + Generated Shadow Polygons Show ArcGIS Pro with building footprints (blue) and shadow polygons (gray, 50% transparent) overlaid. Open the attribute table showing Timestamp and BuildingID columns. How to capture: Run the add-in on NYC Building Footprints (Midtown subset). Style: shadows = #333333 fill, 50% transparency, no outline. Buildings = #CCE5FF fill. Screenshot the map + attribute table together at 1920×1080. Caption: “Shadow polygons stored as a feature class with temporal attributes, ready for publishing.”

Figure 3 — ArcGIS Pro showing building footprints (blue) with computed shadow polygons (gray) stored as a feature class.

### Step 3 — Writing to a Feature Class (ArcGIS Pro SDK)

Shadow polygons for each time step are written to a geodatabase feature class with a Timestamp field. This enables time-based filtering later in the web application.

FeatureWriter.cs — ArcGIS Pro SDK

await QueuedTask.Run(() => { using RowBuffer rowBuffer = shadowFeatureClass.CreateRowBuffer(); using EditOperation editOperation = new EditOperation(); editOperation.Callback(context => { foreach (var (shadowPolygon, timestamp) in shadowResults) { rowBuffer["SHAPE"] = shadowPolygon; rowBuffer["Timestamp"] = timestamp; // DateTime of this shadow rowBuffer["BuildingID"] = buildingId; // Source building reference rowBuffer["ShadowLen"] = shadowLength; // Shadow length in meters shadowFeatureClass.CreateRow(rowBuffer); } context.Invalidate(shadowFeatureClass); }, shadowFeatureClass); await editOperation.ExecuteAsync(); await Project.Current.SaveEditsAsync(); });

### Step 4 — Loading as a FeatureLayer in the JavaScript SDK

Once the shadow dataset is published as a feature service (or exported as GeoJSON), it loads in the Maps SDK for JavaScript as a standard FeatureLayer. Because each shadow feature carries a Timestamp attribute, the TimeSlider widget can filter them by hour — giving users a fully interactive temporal shadow explorer.

main.js — ArcGIS Maps SDK for JavaScript

import SceneView from "@arcgis/core/views/SceneView"; import WebScene from "@arcgis/core/WebScene"; import FeatureLayer from "@arcgis/core/layers/FeatureLayer"; import TimeSlider from "@arcgis/core/widgets/TimeSlider"; // 1. Load shadow data as a standard FeatureLayer const shadowLayer = new FeatureLayer({ url: "https://services.arcgis.com/{org}/FeatureServer/0", timeInfo: { startField: "Timestamp" }, renderer: { type: "simple", symbol: { type: "polygon-3d", symbolLayers: [{ type: "flat-color", color: [30, 30, 30, 0.55] }] } } }); // 2. Add to a 3D scene const view = new SceneView({ container: "viewDiv", map: new WebScene({ portalItem: { id: "YOUR_CITY_SCENE_ID" } }) }); view.map.add(shadowLayer); // 3. TimeSlider — navigate hourly shadow snapshots const timeSlider = new TimeSlider({ container: "timeSliderDiv", view, timeVisible: true, loop: true, playRate: 1000 }); // 4. SQL query on shadow features view.whenLayerView(shadowLayer).then(async lv => { const result = await lv.queryFeatures({ where: "Timestamp >= '2024-06-21 08:00' AND Timestamp < '2024-06-21 09:00'", returnGeometry: true }); console.log(`Shadows at 8 AM: ${result.features.length} features`); });

📸 Figure 3 — Web App — 3D Scene with Shadow FeatureLayer + TimeSlider Show the web app in Chrome: 3D city scene, shadow polygons (dark, semi-transparent), TimeSlider at bottom, popup open. How to capture: Serve WebApp/index.html locally with: npx serve . Press Play on the TimeSlider, pause at 9 AM, click a shadow polygon to show its popup. Screenshot at 1920×1080 showing buildings, shadows, TimeSlider, and popup. Caption: “The shadow FeatureLayer in a 3D SceneView. Each polygon is a real, queryable GIS feature.”

Figure 4 — Time-animated shadow features in a 3D SceneView. Each polygon is queryable and carries temporal attributes.

## Why This Matters for Urban GIS

Converting shadows from a visualization effect into GIS data unlocks planning and analytical capabilities that have never existed in the standard ArcGIS toolset.

| 1 | Regulatory shadow compliance Cities like New York, London, and Amsterdam have shadow zoning laws. With shadow data as features, analysts can run spatial intersections to determine exactly how many hours per day a building casts shadow onto protected spaces — parks, schoolyards, solar panels — and export that as a compliance report. |
| --- | --- |

| 2 | Time-series shadow accumulation Because each hourly shadow is a stored feature, you can union shadows across a full day to compute cumulative shadow coverage as a query result, not just a visual widget. This is directly useful for solar energy site assessment and urban heat island modeling. |
| --- | --- |

| 3 | Cross-platform enterprise reuse Shadow data in a geodatabase or feature service can be consumed by ArcGIS Pro analysis tools, ArcGIS Dashboards, StoryMaps, and any application that reads GeoJSON or a feature service REST endpoint. The visualization is just one consumer of the underlying data. |
| --- | --- |

| 4 | Digital twin integration Shadow datasets can feed into digital twin environments — ArcGIS Urban or CityEngine — as data inputs for energy modeling, microclimate simulation, and pedestrian comfort scoring, not just 3D visuals. |
| --- | --- |

## Limitations and Considerations

|  | Important caveats for production use This workflow produces ground-plane shadow projections from building footprints. It does not account for terrain elevation, existing shadow overlap between buildings, or atmospheric refraction. For regulatory-grade analysis, validate results against ArcGIS Pro's 3D solar tools. |
| --- | --- |

Additional considerations:

- Building height data must be accurate — errors in height produce proportionally incorrect shadow lengths.

- Large cities with thousands of buildings generate very large feature datasets for full-day analysis. Consider tiling by district.

- Shadow overlap between adjacent buildings is not merged in this implementation — union operations add processing time in dense areas.

- The solar position algorithm used here is a simplified analytical model. For sub-degree accuracy, integrate NOAA's Solar Calculator or NREL's Solar Position Algorithm (SPA).

## Future Directions

Several enhancements could make this workflow production-ready:

- GPU-accelerated geometry computation using parallel processing in the Pro SDK for large-city datasets.

- Integration with ArcGIS Data Pipelines to schedule automated daily shadow dataset regeneration.

- 4D streaming using time-aware vector tile layers for web performance at urban scale.

- ArcGIS Urban integration to link shadow analysis directly to zoning rule evaluation.

- Building-to-building shadow attribution — tracking which buildings cast shadows onto which parcels.

## Repository and Getting Started

The full implementation — ArcGIS Pro SDK add-in, JavaScript web app, and sample building footprint data — is available on GitHub.

|  | GitHub Repository https://github.com/ArifulIslamEsri/city-shadow-project |
| --- | --- |

To run the Pro SDK add-in:

- Clone the repository and open ProSDK/CityShading.sln in Visual Studio 2022.

- Ensure ArcGIS Pro SDK for .NET (200.x) is installed.

- Build and run — the add-in appears under Add-In Tab → Shadow Generator in ArcGIS Pro.

- Load a building footprint feature class (e.g., NYC OpenData). Shadow polygons are written to Data/ShadowOutput.gdb.

## Conclusion

Shadow analysis in ArcGIS has always answered the question: "What does this shadow look like?" This project asks a different question: "What can we do with shadow data?"

By shifting shadows from a rendering effect to a persistent, queryable feature layer, we unlock capabilities that standard ArcGIS shadow tools simply do not provide: SQL querying, time-series analysis, spatial joins, enterprise storage, and cross-platform reuse. And with free, high-quality building datasets from NYC Open Data and Google Open Buildings, there is no barrier to getting started.

The ArcGIS Pro SDK for .NET provides the geometry engine to compute shadow polygons programmatically. The ArcGIS Maps SDK for JavaScript provides visualization and query capabilities to consume them. Together, they form a pipeline that moves urban shadow analysis out of the visualization layer and into the data layer — where it can do real analytical work.

|  | Share your thoughts Have you worked with shadow analysis in ArcGIS? Are there regulatory or planning workflows where persistent shadow data would help your organization? Drop a comment on the ArcGIS Community forums or connect on LinkedIn. |
| --- | --- |

Download the add-in from GitHub and start turning your ArcGIS shadows into data today!

| AI | Ariful Islam GIS Developer specializing in ArcGIS Pro SDK and Maps SDK for JavaScript. Passionate about converting geospatial visualization into actionable data workflows for urban planning and digital twin applications. |
| --- | --- |

Tags:  3d visualization & analytics  ·  arcgis pro sdk  ·  maps sdk for javascript  ·  urban gis  ·  shadow analysis  ·  feature layer  ·  developer workflow  ·  c#

### Related Articles

| 3D Visualization · ArcGIS Pro Create a 3D Printable STL with ArcGIS Pro |  | 3D Visualization · Scene Viewer Visualizing Flood Simulations: from ArcGIS Pro to the Web with Flow Rendering |
| --- | --- | --- |

| Local Government · 3D Buildings Create High-Detail Buildings with the Updated 3D Buildings Solution |  | 3D Visualization · ArcGIS Online What's New in Scene Viewer (February 2026) |
| --- | --- | --- |